NNFL – 49275 Autumn 2026

Fuzzy Logic Control of Ball and Beam System

Table of Contents

# Introduction:

Problem statement

project objectives

background

# Literature Review

Summary of related works and approaches.

## 1. Why the Ball and Beam System is Studied

The ball and beam system is a classic benchmark problem in control engineering. A benchmark problem is a standard test system used to evaluate and compare different control strategies. It is popular because:

- It is open-loop unstable (without control, the ball rolls off).

- It is nonlinear (motion depends on gravity and trigonometric terms like sin θ).

- It is underactuated (we control beam angle but want to control ball position).

- It behaves like a fourth-order system, meaning control affects higher-order derivatives of motion.

These properties make it ideal for testing classical, nonlinear, and intelligent control methods.

Early nonlinear modelling work (e.g., Hauser et al., 1992; Ortega et al., 2002) shows the system’s full nonlinear dynamics. Experimental platforms such as the Quanser Ball & Beam (Quanser, 1991) are widely used in laboratories.

## 2. Classical Control: PID and Its Limits

Most laboratories use PID (Proportional–Integral–Derivative) control because it is simple and easy to implement. PID adjusts control effort based on:

- Position error,

- Accumulated error,

- Rate of change of error.

However:

- Stability analysis usually uses a linearized model, not the true nonlinear one.

- Performance worsens for large angles.

- Requires careful tuning.

- Does not handle uncertainties robustly.

Nonlinear PD stability analysis (e.g., Yu & Ortiz, 2005) showed that adding gravity compensation improves stability, but performance remains limited compared to more advanced controllers.

## 3. Sliding Mode and Fractional Controllers

Sliding Mode Control (Hirschorn, 2002) provides strong robustness but causes chattering (high-frequency oscillation).

Fractional-order control (Mártón & Lantos; Oustaloup’s CRONE control) smooths switching using fractional derivatives (Caputo definition). Results show:

- Reduced chattering,

- Better noise rejection,

- More robustness when derivative order β < 1.

However, these methods are mathematically complex.

## 4. Fuzzy Logic Control (FLC)

FLC was introduced to avoid reliance on exact models (Wang, 1998). It mimics human reasoning using:

- Fuzzification – Convert error into linguistic terms (e.g., “Positive Large”).

- Rule Base – IF–THEN rules.

- Inference Engine – Apply logical reasoning.

- Defuzzification – Produce motor voltage output.

Example rule:
IF error is Large Positive AND velocity is Positive → Tilt beam Negative Large.

Why FLC works well here:

Handles nonlinear behaviour naturally.

No need for exact system equations.

Produces smooth control (no chattering).

Easier implementation than nonlinear model-based control.

## 5. FLC vs PID vs Neural Networks

| Feature | PID | FLC | Neural Network (NN) |
| --- | --- | --- | --- |
| Model required | Linear approx. | No | No (data-based) |
| Complexity | Low | Moderate | High |
| Stability proof | Easy (linear) | Hard | Often none |
| Adaptation | No | Limited | Yes |

Neural control (Chu & Huang, 1999; Eaton et al., 2000) learns from data but requires training and may lack theoretical guarantees. FLC provides interpretability and simpler design compared to NN.

## 6. Hardware vs Software Implementation

Software (MATLAB/Simulink)

- Easy simulation of nonlinear model.

- Useful for controller tuning and testing.

- Fractional and fuzzy toolboxes available.

Hardware (Arduino or simple microcontrollers)

- Low-cost real-time implementation.

- Requires simplified rule base.

- Limited computational power (fractional controllers harder).

- FLC is easier than NN or fractional control on Arduino.

## 7. Research Gaps and Future Directions

Current gaps:

- Lack of strong global stability proofs for FLC.

- Manual rule tuning.

- Limited adaptation to parameter changes.

- Integration with low-cost hardware.

Future directions:

- Adaptive FLC – Online tuning of rules.

- Fuzzy–Sliding hybrids – Combine robustness and smoothness.

- Fractional–Fuzzy systems – Improved noise handling.

- AI-based optimization – Use genetic algorithms for tuning.

- Low-cost embedded implementations – Arduino/FPGA-based intelligent control.

The ball and beam system remains a powerful benchmark because it captures instability, nonlinearity, and under-actuation in a simple setup. PID control is simple but limited. Sliding mode and fractional control improve robustness but increase complexity.

Fuzzy Logic Control offers a balanced solution: simple, intuitive, smooth, and effective for nonlinear behaviour. While it lacks strong theoretical guarantees compared to model-based nonlinear control, it remains one of the most practical intelligent control approaches for this benchmark system.

# Aims and Objectives

# System Design

Detailed explanation of the Ball and Beam setup and fuzzy logic controller design.

# Implementation

Describe how the fuzzy logic controller was implemented, including hardware and software.

# Results and Discussion

Analyse system performance, tuning, and testing outcomes.

# Conclusion

Summary of achievements and future work.